Faculty Monitoring System

System Manual

Version: 1.0

Date: May 2026

System: LoRa-Based Faculty Room Monitoring with Real-Time Dashboard

# Table of Contents

1. Introduction

2. System Overview

3. System Architecture

4. Hardware Components

5. Software Components

6. Installation Guide

7. System Configuration

8. Operating Modes

9. User Guide

10. Dashboard Guide

11. Troubleshooting

12. Maintenance

13. Technical Specifications

14. Appendix A — Packet Format

15. Appendix B — Firebase Database Structure

16. Appendix C — Error Codes

# 1. Introduction

The Faculty Monitoring System is an Internet of Things (IoT)-based solution developed to monitor faculty presence and room occupancy in real time. The system uses RFID technology for identification, LoRa wireless communication for long-range data transmission, and Firebase cloud services for centralized data storage and dashboard monitoring.

The system is designed for schools, universities, and institutions that require automated attendance tracking and faculty room monitoring.

# 2. System Overview

## 2.1 Purpose of the System

The system aims to:

- Monitor faculty movement between rooms

- Record attendance automatically

- Track faculty room occupancy

- Provide centralized real-time monitoring

- Reduce manual attendance processes

- Improve monitoring efficiency

## 2.2 Main Features

### Real-Time Monitoring

The dashboard updates faculty locations instantly using Firebase real-time synchronization.

### Long-Range Communication

LoRa technology allows communication between transmitters and receiver up to approximately 2 kilometers in open areas.

### RFID-Based Identification

Each professor uses an RFID card for identification and attendance logging.

### Multi-Room Support

The system supports multiple room transmitters simultaneously.

### Faculty Room Tracking

Multiple professors can be monitored inside the faculty room at the same time.

### Cloud-Based Dashboard

Administrators can monitor all rooms and logs remotely through a web dashboard.

### Registration System

New RFID cards can easily be registered using the registration mode.

## 2.3 Major Components

The system consists of three major parts:

| Component | Description |
| --- | --- |
| Transmitter | Installed in each room to scan RFID cards |
| Receiver | Central hub that receives LoRa data and uploads to Firebase |
| Dashboard | Web application used for monitoring and administration |

# 3. System Architecture

## 3.1 Network Topology

Firebase Cloud

│

WiFi / Internet

│

┌─────────────────┐

│    Receiver     │

│ ESP32 + LoRa    │

│ RFID + OLED     │

└─────────────────┘

│

LoRa 915 MHz

┌──────┬──────┬──────┐

│      │      │      │

Rm101  Rm102  Rm103  Rm104

## 3.2 Data Flow

- Professor taps RFID card on transmitter.

- Transmitter reads RFID UID.

- LoRa packet is sent to receiver.

- Receiver processes the packet.

- Receiver uploads data to Firebase.

- Dashboard updates in real time.

# 4. Hardware Components

## 4.1 Receiver Hardware

### Main Board

Heltec WiFi LoRa 32 V2

| Specification | Details |
| --- | --- |
| Microcontroller | ESP32 Dual-Core |
| Frequency | 240 MHz |
| Flash Memory | 8 MB |
| RAM | 320 KB |
| LoRa Chip | SX1276 |
| WiFi | 2.4 GHz |
| Display | 0.96" OLED |

## 4.2 Additional Receiver Components

| Component | Function |
| --- | --- |
| MFRC522 RFID Reader | Reads RFID cards |
| LoRa Antenna | Wireless communication |
| USB-C Power Supply | Power source |

## 4.3 Transmitter Hardware

Each room transmitter contains:

- ESP32 board

- LoRa module

- MFRC522 RFID reader

- LoRa antenna

- Power supply

## 4.4 Pin Connections

### LoRa Module

| Pin | GPIO |
| --- | --- |
| CS | GPIO 18 |
| RST | GPIO 14 |
| DIO0 | GPIO 26 |

### RFID Reader

| Pin | GPIO |
| --- | --- |
| SS | GPIO 21 |
| RST | GPIO 17 |

### OLED Display

| Pin | GPIO |
| --- | --- |
| SDA | GPIO 4 |
| SCL | GPIO 15 |

# 5. Software Components

## 5.1 Technologies Used

| Technology | Purpose |
| --- | --- |
| Arduino Framework | ESP32 programming |
| Firebase | Cloud database |
| React / Web Dashboard | Monitoring interface |
| PlatformIO | Firmware uploading |
| LoRa Library | Wireless communication |
| MFRC522 Library | RFID communication |

## 5.2 Required Software

### For Embedded Development

- VS Code

- PlatformIO Extension

- Arduino Framework

### For Dashboard

- Node.js

- npm package manager

# 6. Installation Guide

## 6.1 Receiver Installation

### Step 1 — Connect RFID Reader

Connect the RFID reader according to the pin configuration.

Important reminders:

- Use 3.3V only

- Avoid loose wiring

- Keep wires short for stable communication

### Step 2 — Connect LoRa Antenna

Attach the LoRa antenna securely before powering on the board.

⚠️  Never power the device without the antenna attached.

### Step 3 — Connect Power

Power the receiver using:

- USB-C cable

- or 5V DC adapter

## 6.2 Transmitter Installation

### Step 1 — Install Device

Mount the transmitter near the room entrance.

### Step 2 — Configure Room ID

Example:

String roomId = "room_101";

Each room must have a unique room ID.

### Step 3 — Upload Firmware

Upload the configured firmware to the ESP32 transmitter.

## 6.3 Software Setup

### Configure WiFi

Edit:

#define WIFI_SSID "Your_WiFi_Name"

#define WIFI_PASSWORD "Your_WiFi_Password"

### Configure Firebase

Update Firebase database URL:

#define FIREBASE_DATABASE_URL "https://your-project.firebaseio.com"

### Upload Firmware

Using PlatformIO:

platformio run --target upload

## 6.4 Dashboard Setup

### Install Dependencies

npm install

### Start Dashboard

npm start

### Open Dashboard

http://localhost:5200

# 7. System Configuration

## 7.1 WiFi Configuration

The receiver connects to WiFi to upload logs to Firebase.

Requirements:

- 2.4 GHz WiFi

- Stable internet connection

## 7.2 Firebase Configuration

The Firebase database stores:

- Professor information

- Attendance logs

- Room occupancy

- Registration requests

## 7.3 Room Configuration

Each transmitter must have:

- Unique room ID

- Correct LoRa frequency

- Proper antenna connection

# 8. Operating Modes

The receiver supports three operating modes.

## 8.1 LoRa Monitoring Mode

### Purpose

Receive attendance data from transmitters.

### Functions

- Receives LoRa packets

- Displays packet information

- Uploads data to Firebase

- Updates dashboard in real time

## 8.2 Faculty Room Mode

### Purpose

Track professors entering and leaving the faculty room.

### Process

- Tap RFID card once → IN

- Tap again → OUT

Multiple occupants are supported simultaneously.

## 8.3 Registration Mode

### Purpose

Register new RFID cards.

### Process

- Enable registration mode

- Scan RFID card

- UID sent to Firebase

- Admin approves card

- Professor details are added

## 8.4 Switching Modes

### Method 1 — Physical Button

Hold the PROG button for 2 seconds.

Mode sequence:

- LoRa Monitoring

- Faculty Room

- Registration

- LoRa Monitoring (cycle)

### Method 2 — Admin RFID Card

Admin RFID card can toggle Registration Mode.

### Method 3 — Dashboard Control

The dashboard can remotely enable or disable Registration Mode.

# 9. User Guide

## 9.1 For Professors

### Entering a Room

- Approach room transmitter

- Tap RFID card

- Wait for confirmation

- Attendance is recorded automatically

### Using Faculty Room

- Approach receiver

- Ensure Faculty Room Mode is active

- Tap RFID card

- Status toggles automatically

## 9.2 For Administrators

### Registering New Cards

Step 1 — Enable Registration Mode

Use:

- PROG button

- Admin card

- Dashboard

Step 2 — Scan Card

Professor taps RFID card.

Step 3 — Approve Registration

Approve the card in dashboard registration page.

Step 4 — Add Professor Information

Input:

- Name

- Department

- Email

### Managing Professors

Administrators can:

- Edit professor information

- Delete professor accounts

- Manage whitelist access

# 10. Dashboard Guide

## 10.1 Monitoring Dashboard

The dashboard displays:

- Room occupancy

- Faculty locations

- Attendance logs

- Real-time statistics

## 10.2 Registration Page

Features include:

- Pending registrations

- Manual registration

- Registered professor list

## 10.3 Admin Panel

The admin panel provides:

- System statistics

- User management

- Logs monitoring

- Configuration controls

# 11. Troubleshooting

## 11.1 Receiver Not Receiving LoRa Packets

### Possible Causes

- Antenna disconnected

- Wrong frequency

- Long distance

- Incorrect mode

### Solutions

- Check antenna connection

- Ensure 915 MHz frequency

- Move devices closer

- Switch to LoRa Monitoring Mode

## 11.2 WiFi Connection Failure

### Solutions

- Verify WiFi credentials

- Ensure 2.4 GHz WiFi

- Restart receiver

- Check signal strength

## 11.3 RFID Reader Not Working

### Solutions

- Verify wiring

- Check 3.3V supply

- Restart device

- Test RFID card

## 11.4 Firebase Upload Failure

### Solutions

- Verify internet connection

- Check Firebase credentials

- Check Firebase rules

- Restart receiver

## 11.5 Dashboard Not Updating

### Solutions

- Refresh browser

- Check Firebase connection

- Verify receiver uploads

- Inspect browser console

# 12. Maintenance

## 12.1 Daily Maintenance

- Check OLED indicators

- Verify WiFi connection

- Test RFID reader

## 12.2 Weekly Maintenance

- Review dashboard logs

- Check all transmitters

- Monitor Firebase usage

## 12.3 Monthly Maintenance

- Clean RFID readers

- Inspect antennas

- Archive old logs

- Update firmware

# 13. Technical Specifications

## 13.1 LoRa Specifications

| Parameter | Value |
| --- | --- |
| Frequency | 915 MHz |
| Bandwidth | 125 kHz |
| Spreading Factor | 7 |
| Coding Rate | 4/5 |
| TX Power | 20 dBm |

## 13.2 RFID Specifications

| Parameter | Value |
| --- | --- |
| Frequency | 13.56 MHz |
| Protocol | ISO/IEC 14443A |
| Read Range | 0–10 cm |

## 13.3 Performance

| Feature | Value |
| --- | --- |
| Queue Capacity | 50 packets |
| RFID Read Time | < 500 ms |
| WiFi Reconnect | Automatic |
| Anti-Bounce Delay | 5 seconds |

## 13.4 System Capacity

| Feature | Capacity |
| --- | --- |
| Maximum Transmitters | 50+ |
| Maximum Professors | 500+ |
| Dashboard Users | Unlimited |

# 14. Appendix A — Packet Format

## RFID Packet Format

RFID:<UID>:<timestamp>:<status>:<room_id>

## Example

RFID:A3:4F:2E:91:1234567890:IN:room_202

# 15. Appendix B — Firebase Database Structure

{

"professors": {},

"admins": {},

"monitoring_logs": {},

"faculty_room": {},

"rooms": {},

"pending_registrations": {},

"system_config": {}

}

# 16. Appendix C — Error Codes

| Error Code | Description |
| --- | --- |
| E001 | LoRa initialization failed |
| E002 | WiFi connection timeout |
| E003 | Firebase authentication failed |
| E004 | RFID reader failure |
| E005 | Queue overflow |
| E006 | Invalid packet format |